Load and Store Group

| LDA | Load Accumulator | Absolute | LDA $aaaa | $AD | 3 | N,Z |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Zero Page | LDA $aa | $A5 | 2 |  |
|  |  | Immediate | LDA #$aa | $A9 | 2 |  |
|  |  | Absolute Indexed, X | LDA $aaaa,X | $BD | 3 |  |
|  |  | Absolute Indexed, Y | LDA $aaaa,Y | $B9 | 3 |  |
|  |  | Zero Page Indexed, X | LDA $aa,X | $B5 | 2 |  |
|  |  | Indexed Indirect | LDA ($aa,X) | $A1 | 2 |  |
|  |  | Indirect Indexed | LDA ($aa),Y | $B1 | 2 |  |
| LDX | Load X Register | Absolute | LDX $aaaa | $AE | 3 | N,Z |
|  |  | Zero Page | LDX $aa | $A6 | 2 |  |
|  |  | Immediate | LDX #$aa | $A2 | 2 |  |
|  |  | Absolute Indexed, Y | LDX $aaaa,Y | $BE | 3 |  |
|  |  | Zero Page Indexed, Y | LDX $aa,Y | $B6 | 2 |  |
| LDY | Load Y Register | Absolute | LDY $aaaa | $AC | 3 | N,Z |
|  |  | Zero Page | LDY $aa | $A4 | 2 |  |
|  |  | Immediate | LDY #$aa | $A0 | 2 |  |
|  |  | Absolute Indexed, X | LDY $aaaa,X | $BC | 3 |  |
|  |  | Zero Page Indexed, X | LDY $aa,X | $B4 | 2 |  |
| STA | Store Accumulator | Absolute | STA $aaaa | $8D | 3 | none |
|  |  | Zero Page | STA $aa | $85 | 2 |  |
|  |  | Absolute Indexed, X | STA $aaaa,X | $9D | 3 |  |
|  |  | Absolute Indexed, Y | STA $aaaa,Y | $99 | 3 |  |
|  |  | Zero Page Indexed, X | STA $aa,X | $95 | 2 |  |
|  |  | Indexed Indirect | STA ($aa,X) | $81 | 2 |  |
|  |  | Indirect Indexed | STA ($aa),Y | $91 | 2 |  |
| STX | Store X Register | Absolute | STX $aaaa | $8E | 3 | none |
|  |  | Zero Page | STX $aa | $86 | 2 |  |
|  |  | Zero Page Indexed, Y | STX $aa,Y | $96 | 2 |  |
| STY | Store Y Register | Absolute | STY $aaaa | $8C | 3 | none |
|  |  | Zero Page | STY $aa | $84 | 2 |  |
|  |  | Zero Page Indexed, X | STY $aa,X | $94 | 2 |  |

Arithmetic Group

| ADC | Add with Carry | Absolute | ADC $aaaa | $6D | 3 | N,V,Z,C |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Zero Page | ADC $aa | $65 | 2 |  |
|  |  | Immediate | ADC #$aa | $69 | 2 |  |
|  |  | Absolute Indexed, X | ADC $aaaa,X | $7D | 3 |  |
|  |  | Absolute Indexed, Y | ADC $aaaa,Y | $79 | 3 |  |
|  |  | Zero Page Indexed, X | ADC $aa,X | $75 | 2 |  |
|  |  | Indexed Indirect | ADC ($aa,X) | $61 | 2 |  |
|  |  | Indirect Indexed | ADC ($aa),Y | $71 | 2 |  |
| SBC | Subtract with | Absolute | SBC $aaaa | $ED | 3 | N,V,Z,C |
|  | Carry | Zero Page | SBC $aa | $E5 | 2 |  |
|  |  | Immediate | SBC #$aa | $E9 | 2 |  |
|  |  | Absolute Indexed, X | SBC $aaaa,X | $FD | 3 |  |
|  |  | Absolute Indexed, Y | SBC $aaaa,Y | $F9 | 3 |  |
|  |  | Zero Page Indexed, X | SBC $aa,X | $F5 | 2 |  |
|  |  | Indexed Indirect | SBC ($aa,X) | $E1 | 2 |  |
|  |  | Indirect Indexed | SBC ($aa),Y | $F1 | 2 |  |

Increment and Decrement Group

| INC | Increment a | Absolute | INC $aaaa | $EE | 3 | N,Z |
| --- | --- | --- | --- | --- | --- | --- |
|  | memory location | Zero Page | INC $aa | $E6 | 2 |  |
|  |  | Absolute Indexed, X | INC $aaaa,X | $FE | 3 |  |
|  |  | Zero Page Indexed, X | INC $aa,X | $F6 | 2 |  |
| INX | Increment the X register | Implied | INX | $E8 | 1 | N,Z |
| INY | Increment the Y register | Implied | INY | $C8 | 1 | N,Z |
| DEC | Decrement a | Absolute | DEC $aaaa | $CE | 3 | N,Z |
|  | memory location | Zero Page | DEC $aa | $C6 | 2 |  |
|  |  | Absolute Indexed, X | DEC $aaaa,X | $DE | 3 |  |
|  |  | Zero Page Indexed, X | DEC $aa,X | $D6 | 2 |  |
| DEX | Decrement the X register | Implied | DEX | $CA | 1 | N,Z |
| DEY | Decrement the Y register | Implied | DEY | $88 | 1 | N,Z |

Register Transfer Group

| TAX | Transfer accumulator to X | Implied | TAX | $AA | 1 | N,Z |
| --- | --- | --- | --- | --- | --- | --- |
| TAY | Transfer accumulator to Y | Implied | TAY | $A8 | 1 | N,Z |
| TXA | Transfer X to accumulator | Implied | TXA | $8A | 1 | N,Z |
| TYA | Transfer Y to accumulator | Implied | TYA | $98 | 1 | N,Z |

Logical Group

| AND | Logical AND | Absolute | AND $aaaa | $2D | 3 | N,Z |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Zero Page | AND $aa | $25 | 2 |  |
|  |  | Immediate | AND #$aa | $29 | 2 |  |
|  |  | Absolute Indexed, X | AND $aaaa,X | $3D | 3 |  |
|  |  | Absolute Indexed, Y | AND $aaaa,Y | $39 | 3 |  |
|  |  | Zero Page Indexed, X | AND $aa,X | $35 | 2 |  |
|  |  | Indexed Indirect | AND ($aa,X) | $21 | 2 |  |
|  |  | Indirect Indexed | AND ($aa),Y | $31 | 2 |  |
| EOR | Exclusive OR | Absolute | EOR $aaaa | $4D | 3 | N,Z |
|  |  | Zero Page | EOR $aa | $45 | 2 |  |
|  |  | Immediate | EOR #$aa | $49 | 2 |  |
|  |  | Absolute Indexed, X | EOR $aaaa,X | $5D | 3 |  |
|  |  | Absolute Indexed, Y | EOR $aaaa,Y | $59 | 3 |  |
|  |  | Zero Page Indexed, X | EOR $aa,X | $55 | 2 |  |
|  |  | Indexed Indirect | EOR ($aa,X) | $41 | 2 |  |
|  |  | Indirect Indexed | EOR ($aa),Y | $51 | 2 |  |

| ORA | Logical Inclusive | Absolute | ORA $aaaa | $0D | 3 | N,Z |
| --- | --- | --- | --- | --- | --- | --- |
|  | OR | Zero Page | ORA $aa | $05 | 2 |  |
|  |  | Immediate | ORA #$aa | $09 | 2 |  |
|  |  | Absolute Indexed, X | ORA $aaaa,X | $1D | 3 |  |
|  |  | Absolute Indexed, Y | ORA $aaaa,Y | $19 | 3 |  |
|  |  | Zero Page Indexed, X | ORA $aa,X | $15 | 2 |  |
|  |  | Indexed Indirect | ORA ($aa,X) | $01 | 2 |  |
|  |  | Indirect Indexed | ORA ($aa),Y | $11 | 2 |  |

Compare and Bit Test Group

| CMP | Compare | Absolute | CMP $aaaa | $CD | 3 | N,Z,C |
| --- | --- | --- | --- | --- | --- | --- |
|  | accumulator | Zero Page | CMP $aa | $C5 | 2 |  |
|  |  | Immediate | CMP #$aa | $C9 | 2 |  |
|  |  | Absolute Indexed, X | CMP $aaaa,X | $DD | 3 |  |
|  |  | Absolute Indexed, Y | CMP $aaaa,Y | $D9 | 3 |  |
|  |  | Zero Page Indexed, X | CMP $aa,X | $D5 | 2 |  |
|  |  | Indexed Indirect | CMP ($aa,X) | $C1 | 2 |  |
|  |  | Indirect Indexed | CMP ($aa),Y | $D1 | 2 |  |
| CPX | Compare X | Absolute | CPX $aaaa | $EC | 3 | N,Z,C |
|  | register | Zero Page | CPX $aa | $E4 | 2 |  |
|  |  | Immediate | CPX #$aa | $E0 | 2 |  |
| CPY | Compare Y | Absolute | CPY $aaaa | $CC | 3 | N,Z,C |
|  | register | Zero Page | CPY $aa | $C4 | 2 |  |
|  |  | Immediate | CPY #$aa | $C0 | 2 |  |
| BIT | Bit Test | Absolute | BIT $aaaa | $2C | 3 | N,V,Z |
|  |  | Zero Page | BIT $aa | $24 | 2 |  |

Shift and Rotate Group

| ASL | Arithmetic Shift | Absolute | ASL $aaaa | $0E | 3 | N,Z,C |
| --- | --- | --- | --- | --- | --- | --- |
|  | Left | Zero Page | ASL $aa | $06 | 2 |  |
|  |  | Accumulator | ASL A | $0A | 1 |  |
|  |  | Absolute Indexed, X | ASL $aaaa,X | $1E | 3 |  |
|  |  | Zero Page Indexed, X | ASL $aa,X | $16 | 2 |  |
| LSR | Logical Shift | Absolute | LSR $aaaa | $4E | 3 | N,Z,C |
|  | Right | Zero Page | LSR $aa | $46 | 2 |  |
|  |  | Accumulator | LSR A | $4A | 1 |  |
|  |  | Absolute Indexed, X | LSR $aaaa,X | $5E | 3 |  |
|  |  | Zero Page Indexed, X | LSR $aa,X | $56 | 2 |  |

| ROL | Rotate Left | Absolute | ROL $aaaa | $2E | 3 | N,Z,C |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Zero Page | ROL $aa | $26 | 2 |  |
|  |  | Accumulator | ROL A | $2A | 1 |  |
|  |  | Absolute Indexed, X | ROL $aaaa,X | $3E | 3 |  |
|  |  | Zero Page Indexed, X | ROL $aa,X | $36 | 2 |  |
| ROR | Rotate Right | Absolute | ROR $aaaa | $6E | 3 | N,Z,C |
|  |  | Zero Page | ROR $aa | $66 | 2 |  |
|  |  | Accumulator | ROR A | $6A | 1 |  |
|  |  | Absolute Indexed, X | ROR $aaaa,X | $7E | 3 |  |
|  |  | Zero Page Indexed, X | ROR $aa,X | $76 | 2 |  |

Jump and Branch Group

| JMP | Jump to another location | Absolute | JMP $aaaa | $4C | 3 | none |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Indirect | JMP ($aaaa) | $6C | 3 |  |
| BCC | Branch if carry flag clear | Relative | BCC aa | $90 | 2 | none |
| BCS | Branch if carry flag set | Relative | BCS aa | $B0 | 2 | none |
| BEQ | Branch if zero flag set | Relative | BEQ aa | $F0 | 2 | none |
| BMI | Branch if negative flag set | Relative | BMI aa | $30 | 2 | none |
| BNE | Branch if zero flag clear | Relative | BNE aa | $D0 | 2 | none |
| BPL | Branch if negative flag clear | Relative | BPL aa | $10 | 2 | none |
| BVC | Branch if overflow flag clear | Relative | BVC aa | $50 | 2 | none |
| BVS | Branch if overflow flag set | Relative | BVS aa | $70 | 2 | none |

Stack Group

| TSX | Transfer stack pointer to X | Implied | TSX | $BA | 1 | N,Z |
| --- | --- | --- | --- | --- | --- | --- |
| TXS | Transfer X to stack pointer | Implied | TXS | $9A | 1 | none |
| PHA | Push accumulator on stack | Implied | PHA | $48 | 1 | none |
| PHP | Push processor status on stack | Implied | PHP | $08 | 1 | none |
| PLA | Pull accumulator from stack | Implied | PLA | $68 | 1 | N,Z |
| PLP | Pull processor status from stack | Implied | PLP | $28 | 1 | All |

Status Flag Change Group

| CLC | Clear carry flag | Implied | CLC | $18 | 1 | C |
| --- | --- | --- | --- | --- | --- | --- |
| CLD | Clear decimal mode flag | Implied | CLD | $D8 | 1 | D |
| CLI | Clear interrupt disable flag | Implied | CLI | $58 | 1 | I |
| CLV | Clear overflow flag | Implied | CLV | $B8 | 1 | V |
| SEC | Set carry flag | Implied | SEC | $38 | 1 | C |
| SED | Set decimal mode flag | Implied | SED | $F8 | 1 | D |
| SEI | Set interrupt disable flag | Implied | SEI | $78 | 1 | I |

Subroutine and Interrupt Group

| JSR | Jump to a subroutine | Absolute | JSR $aaaa | $20 | 3 | none |
| --- | --- | --- | --- | --- | --- | --- |
| RTS | Return from subroutine | Implied | RTS | $60 | 1 | none |
| BRK | Force an interrupt | Implied | BRK | $00 | 1 | B, I |
| RTI | Return from Interrupt | Implied | RTI | $40 | 1 | All |
| NOP | No Operation | Implied | NOP | $EA | 1 | none |