Using The I/O Screen For Output

Memory Mapped Input/Output

The 6502 processor uses memory mapped input and output.  This means that there are no registers or  memory buffers dedicated to transferring data in and out of the processor chip.  There is no input / output interface built into the processor chip.  There are no instructions which will transfer data directly to or from a specified i/o device.

Instead, for each input or output device, a set of memory addresses will be set aside as a buffer area, and they will be connected directly to the i/o device.  Let’s say that memory locations $A000 - $AFFF were set aside as the video display buffer.  This means that the motherboard would contain some physical connections between those memory chips and the VDU output port.  As far as the processor is concerned, it is storing some values in the RAM, using the standard STA (Store Accumulator in memory) instructions, but the hardware connections will cause that data to be transmitted directly to the VDU.

In other processors ( and indeed in later models of the 6502 processor) there might be some dedicated i/o registers, and as well as STA (Store Accumulator in Memory), you might have instructions such as TAO (Transfer Accumulator to Output register) and TIA (Transfer Input register to Accumulator).

The Simulator I/O Screen

Given that we are not using a real 6502 processor, but a software simulation of one, we are going to keep life simple by simulating some hardware – a small text only terminal screen.  You can make it visible by clicking on (View / Input output) or by pressing Alt + F5.

Unlike real memory mapped hardware, you can set the terminal screen memory buffer to start at any address you like, by going to the (Simulator / Options) menu.  The default setting is $E000, and all of the example programs will assume that this value is being used.

The following memory location have these functions:

$E000	Clear terminal window

$E001	Ascii code will be sent to terminal screen and displayed as a character

$E002	Ascii code will be sent to screen  and displayed  as above, but CR/LF will be ignored

$E003	Ascii code will be sent to screen and displayed as a hexadecimal number

$E004	If a character has been typed into the terminal screen, it will be copied to this location

$E005	Controls horizontal location of cursor

$E006	Controls vertical location of cursor.

Any character stored in $E000 will cause the terminal screen to be cleared.

This is only a simulation, so the only ASCII control characters that the terminal screen understands are the following:

Line Feed	ASCII $0A	Go to next line.  Column position not changed. Scroll if last line.

Carriage Return	ASCII $0D	Move cursor to first column. Stay on same line.

Backspace	ASCII $08	Move back one space and erase character.

Using the i/o screen for input will be covered later.

; Backspace.65s

.ORG = $200

io_putc	= $e001

io_posx	= $e005

io_posy	= $e006

LDA #9

STA io_posx		; This will send cursor to column 9

LDA #2

STA io_posy		; This will send cursor to row 2

; print 'aaa'

LDA #'a'

STA io_putc

STA io_putc

STA io_putc

; print backspace three times erasing a's

LDA #8

STA io_putc

STA io_putc

STA io_putc

BRK

Step through the program above and see it working.

Notes

Note the use of constants at the top of the program, to make it easier to refer to the various i/o addresses.  The program would work equally well if you had lines like STA $E001, but the constants make it more readable.

Note the use of decimal numbers in the operands, such as LDA #9.  I just wanted to show it being done.  I would normally used LDA #$09, which means exactly the same thing:

Put the number 9 (immediate operand) into the accumulator register

When writing assembler code, it is often easier to think in hex, because you are manipulating data at the binary level, and it is easier to go from binary to hex than it is from binary to decimal.

Having said that, being able to use ‘a’, instead of having to look up the ASCII code ( $61 ) is quite useful.

Exercise

Write a program which will output something like this on the i/o screen.

1

2

3

*

*

*

*