Quick start instruction

ap2222pc.com is .com file runs in pc

ap2222pc.dsk is the disk image to run on apple

to load whole program to your apple

1. connect lines according to file lineconn.doc

2. boot your apple ][ with a 48k dos 3.3 disk

3. on your apple ][ ,type codes in file loader.asm

4. run ap2222pc.com at your pc as follows:

"ap2222pc -2 ap2222pc.dsk"

don't press any key when you see "DOIT" on pc screen

5. insert a formatted disk in apple ][ disk drive S6 D1

WARNING! THE INFO ON THE DISK WILL LOST FOREVER!

now run the code in apple..."300G (return)"

6. now hit any key on pc side to start

7. you can see apple disk drive is in action...wait all things stop

don't hit any key on your pc or it will quit

8. check if the checksum on both screen is match

NOTICE!on pc side is decimal number (should be 226)

on apple side is HEX number  (should be E2)

9. now you can use the whole program...please read ap2222pc.doc