this file contains info to connect lines between pc and apple ][

1.apple ][ game i/o pins

to keyboard side               as an apple ][ user, you'll see

^^^^^^^^^^^

���Ŀ ���Ŀ                            ����������Ŀ

+5  �1  ��� 16� none                  none � 9       8� GND

PB0 �2      15� AN0                    GC1 �10       7� GC2

PB1 �3      14� AN1                    GC3 �11       6� GC0

PB2 �4      13� AN2                    AN3 �12       5� strobe

strobe �5      12� AN3                    AN2 �13       4� PB2

GC0 �6      11� GC3                    AN1 �14       3� PB1

GC2 �7      10� GC1                    AN0 �15       2� PB0

GND �8      9 � none                  none �16 ��Ŀ  1� +5

�����������                            �����  �����

since keyboard is in

back board side                         this direction

2.pc parallel port 25-pin

please buy a 25-pin "jump wire connector"

(for rs-232 is ok , they both are 25-pin)

�\

� \

1 �  \

2 �   \

3 �    � 14

4 �    � 15

5 �    � 16

6 �    � 17

7 �    � .

8 �    � .

9 �    � .

10 �    � .

11 �    � 25

12 �   /

13 �  /

� /

�/

3.line connection:  9 lines total

apple --- pc

GND     pin 25         as GND

PB0     pin 2          as apple side handshake in (apple -> pc);data bit 0

PB1     pin 3          apple input data bit 1

PB2     pin 4          apple input data bit 2

GC0     pin 5          as apple side handshake in (pc -> apple)

AN0     pin 15         apple output data bit 0

AN1     pin 13         apple output data bit 1

AN2     pin 12         apple output data bit 2

AN3     pin 10         apple side handshake out

I use a "breadboard" , a jump wire connector and a 16-pin cable and 9 wire