********************************************************************************

apple <-> pc disk and sequential text file transferring program

Clay Chang,Taipei,Taiwan,ROC OCT-1994

********************************************************************************

1.Who will need this?

Those who want to transfer their apple ][ disks to pc in .dsk format and vice

versa but don't own a serial card

Those who want to transfer sequential files between apple ][ and pc but don't

own a serial card.

2.System Requirement

apple ][+ 48k (and better apple ][s) with 16 pin game I/O,one disk drive

apple DOS 3.3 boot disk

48K DOS! ... don't use 64K dos or ProDos

PC with parallel port

3.Function Description

This version of program runs under DOS 3.3,not on ProDos

it uses pc parallel port #1 (PRN or LPT1),auto seek port address

1.apple ][ disk -> pc .dsk file

transfer a standard DOS 3.3 RWTS readable disk (5.25") to pc in .dsk form

you can use the .dsk file with apl2em  only 35 tracks now

(almost all unprotected 16-sectors disks,include prodos formatted disk)

2.pc .dsk file -> apple ][ disk

transfer .dsk file to a formatted apple ][ 5.25" floopy disk

all things on the floopy will be destoryed

thus you can play things on real apple ][

3.apple ][ sequential text file -> pc sequential text file

transfer sequential text file to pc

you can transfer docs,programs...etc(even binsciied files)

all control chars will be eliminated

CR -> CR-LF

4.pc sequential text file -> apple ][ sequential text file

transfer sequential text file to apple

you can transfer .bsc .exe and .bas to your apple

may need transfer to ProDos disk by your own if you want to sciibin and

unshrink it (and EXEC)

all control chars will be eliminated

CR-LF -> CR

4.Limitation

1.must boot with DOS 3.3 on apple side (say, not for ProDos)

2.not very fast speed(?)

3.35 track disks only now

4.unprotected standard-RWTS-able-to-handle disks only now

5.Performance

on my 286-12 ancient machine, it takes 7 minute to transfer a disk

(it runs apl2em much slower than real apple ][)

it WILL run much faster on a better pc,trust me!

6.Operating method

prevent damaging important data,please use backup disks to work!

apple side:

transfer ap2222pc.dsk to an apple ][ disk according to file quickin.doc

after boot with DOS 3.3, insert ap2222pc.dsk disk and "RUN WORK"

every once you re-run the program,you must insert the disk contain the files

(it will BLOAD subroutines each RUN)

pc side:

"ap2222pc -<option> <filename>" to work

where <option> is one of following:

1  apple -> pc disk transfer

2  pc -> apple disk transfer

3  apple -> pc text file transfer

4  pc -> apple text file transfer

<filename> could contain drive,path and filename itself

example: "ap2222pc -1 disk6a.dsk"

"ap2222pc -2 ap2222pc.dsk"

"ap2222pc -3 c:/docs/apple.txt"

"ap2222pc -4 c:/docs/apple.txt"

NOTICE:please following instructions on both screen to press keys to work

7.Technical data (simple)

apple game i/o port

3 one digit input (PB0~2)

1 strobe output

(could be used with a flip-flop to get constant voltage signal eg.74LS74)

4 one digit output (AN0~3)

4 analgue input (GC0~3)

pc parallel port

some multi i/o card permit parallel port bidirection data transmission

if your port address is $378 , then

$378 is data out port (usually outout only)

bit  7  6  5  4  3  2  1  0

pin  9  8  7  6  5  4  3  2

$379 is status port (input only)

bit  7  6  5  4  3  2  1  0

pin  11 10 12 13 15 x  x  x

you can use MFT to check your parallel port address

apple printer card (for parallel printers)

as long as you grounded pin 2 and 20 (busy and GND)

every key stroke (or STA $C090) will send data to pin 3,5,7,9,11,13,15,17

and auto send a strobe signal at pin 1

busy could be as handshake in (really?)

and strobe could be as handshake out

8.Using the subroutine

those codes with source is inside ap2222pc.dsk

atpdsk.asm located $9200, transfer disks from apple to pc

ptadsk.asm located $9300, transfer disks from pc to apple

file.asm   located $300, 2 subroutine for using game i/o input & output

3 bits once if data is 12345678

then apple out is 123 456 780  order (pc in)

apple in  is 012 345 678  order (pc out)

$9500~$95FF is buffer for one sector i/o

$07 stores checksum

$08 stores data read/write

9.Concepts dealed

general:parallel data transmission,handshakes i/o,text file structure,checksum

apple side:game i/o,RWTS,assembly,text file r/w,

pc side:parallel port i/o,block and sequential file,command line tail

memory allocate,bios data seeking,dos file handler,F-PC & TCOM

10.No responsibility & about copy rights

The author will not responsible for any damage or any other consequences

of using this program,don't find me if any disk ruined.

This program is virus-free at the time author spread out

You can freely copy and distribute this archive as long as the content

isn't changed.you can't sell this program or earn money with this program

11.Thanks to

the creators of apple ][,DOS 3.3 and ProDos

the creator of F-PC and TCOM

all of you using this program

12.future plans

duo to study works,next reversion time will be summer of 1995

disk <-> .dsk operation under ProDos

(use MLI call,.dsk file require interleave)

sequential text file transfer under ProDos

(use bload & bsave and more assembly)

modulize,optional use 4 bit (with strobe) or apple printer card

pc 2-way parallel port

read/write disk in nibble form,handle protected disks(how?I dunno yet)

13.files contain in the zip archive

ap2222pc.seq     source code of ap2222pc.com , must compile with newest TCOM

ap2222pc.com     pc side program

ap2222pc.dsk     apple side program in disk image form

include assembly codes and sources and basic program

ap2222pc.doc     this doc

readme.1st       what you should read 1st

lineconn.doc     line connection doc

loader.asm       apple side code to load all program (pc .dsk ->apple disk)

quickin.doc      quick start instructions

14.Feedback to the author

You don't have to mail $$ to me,but please e-mail me back about the

bugs,performance and your feeling about this program

Clay Chang,

Dept. Mechanical Engineering, National Central Univ. 320 ChungLi,Taiwan,ROC

u332302@twncu865.ncu.edu.tw   <--- please e-mail to this place! :)

u339589@sparc20.ncu.edu.tw

(before Sep-1995)